Testing Protocol:

(Version 1.1, 18 October 2001)

Setup:

- Insure Glock stock barrel is installed in weapon.

- Fix weapon in the rest.

- Set aim point on target and set clamps.

- Set target.

Glock Stock Barrel:

- Fire settling rounds to set rest (non-test ammunition).

- Load one magazine with 6 rounds. Load 3 magazines with 5 rounds each.

- Insert 6 round magazine into weapon and carefully rack slide.

- Fire 5 round group.  Check group.

- Drop magazine and replace with next 5 round magazine.

- Repeat previous two steps (changing/pasting target between groups) until grouping is valid (no obvious vertical dispersion due to weapon shift in inserts.

- Drop last magazine, leaving a round chambered.

- Load one magazine with 6 rounds and four magazines with 5 rounds each of barrel test ammunition.

- Set new target with target center adjusted for average group center.  Mark corners on backer.

- Insert 6 round magazine into weapon.

- Fire one round (last settling round) to load first testing round.  Mark round on target to distinguish from test group rounds.

- Fire five round group.  Set new target with group number annotated using corner marks.

- Repeat previous step 4 additional times.

- Collect all five targets and document barrel/ammo/time/date.

- Carefully lock slide to the rear, ejecting last round.

- Ease slide forward on an empty chamber and drop last magazine.

- Pull the trigger.

KKM Precision G22D1 Barrel:

- Install KKM barrel.

- Carefully remove slide from weapon leaving frame installed in rest.

- Disassemble and thoroughly clean slide.

- Reassemble slide with KKM barrel.

- Carefully replace slide on frame.

- Fire settling rounds to set rest (non-test ammunition).

- Load one magazine with 6 rounds. Load 3 magazines with 5 rounds each.

- Insert 6 round magazine into weapon and carefully rack slide.

- Fire 5 round group.  Check group.

- Drop magazine and replace with next 5 round magazine.

- Repeat previous two steps (changing/pasting target between groups) until grouping is valid (no obvious vertical dispersion due to weapon shift in inserts.

- Drop last magazine, leaving a round chambered.

- Load one magazine with 6 rounds and four magazines with 5 rounds each of barrel test ammunition.

- Set new target using corner marks.

- Insert 6 round magazine into weapon.

- Fire one round (last settling round) to load first testing round.  Mark target to distinguish from test group rounds.

- Fire five round group.  Set new target with group number annotated using corner marks.

- Repeat previous step 4 additional times.

- Collect all five targets and document barrel/ammo/time/date.

- Carefully lock slide to the rear, ejecting last round.

- Ease slide forward on an empty chamber and drop last magazine.

- Pull the trigger.

Cautery Custom G22 Barrel:

- Install Cautery Custom G22 Barrel.

- Carefully remove slide from weapon leaving frame installed in rest.

- Disassemble and thoroughly clean slide.

- Reassemble slide with Cautery barrel.

- Carefully replace slide on frame.

- Fire settling rounds to set rest (non-test ammunition).

- Load one magazine with 6 rounds. Load 3 magazines with 5 rounds each.

- Insert 6 round magazine into weapon and carefully rack slide.

- Fire 5 round group.  Check group.

- Drop magazine and replace with next 5 round magazine.

- Repeat previous two steps (changing/pasting target between groups) until grouping is valid (no obvious vertical dispersion due to weapon shift in inserts.

- Drop last magazine, leaving a round chambered.

- Load one magazine with 6 rounds and four magazines with 5 rounds each of barrel test ammunition.

- Set new target using corner marks.

- Insert 6 round magazine into weapon.

- Fire one round (last settling round) to load first testing round.  Mark target to distinguish from test group rounds.

- Fire five round group.  Set new target with group number annotated using corner marks.

- Repeat previous step 4 additional times.

- Collect all five targets and document barrel/ammo/time/date.

- Carefully lock slide to the rear, ejecting last round.

- Ease slide forward on an empty chamber and drop last magazine.

- Pull the trigger.

Custom Barrel Load Test (time/weather permitting)

Setup:

- Insure Cautery Custom G22 barrel is installed in weapon.

- Fix/verify weapon in the rest.

- Set aim point on target and set/check clamps.

- Set target.

Load Workup @ 1.140” OAL:  (Note: grouping is secondary consideration to safety/pressure analysis, so group to group lead will not be maintained. Instead, 2 groups of 5 rounds each will be fired with progressively higher powder charges, beginning at 5.1 gr. and continuing to 6.0 gr. or until excessive pressure signs are observed.)

- Fire settling rounds to set rest (non-test ammunition).

- Load one magazine with 6 rounds. Load 3 magazines with 5 rounds each.

- Insert 6 round magazine into weapon and carefully rack slide.

- Fire 5 round group.  Check group.

- Drop magazine and replace with next 5 round magazine.

- Repeat previous two steps (changing/pasting target between groups) until grouping is valid (no obvious vertical dispersion due to weapon shift in inserts.

- Drop last magazine, leaving a round chambered.

- Load magazines with 10 rounds each (1 magazine per powder charge).

- Insert first magazine into weapon.

- Fire one round (last settling round) to load first testing round.  Mark round.

- Set new target using corner marks.

- Fire ten round group.

- Repeat previous 2 steps until all groups are fired.

- Collect targets and document barrel/ammo/time/date.

End of Session:

- Clear weapon, remove from rest, and secure.

- Unclamp rest.

- Inventory equipment.

- Load equipment in vehicle.

- Depart range.